Computer Organization 2017

HOMEWORK II

系級: 資訊108       學號: F74045018      姓名: 廖其忻

實驗結果圖(snapshot of result)

指令波型圖( Instructions waveform )

選擇的指令：

20041000  //addi $4, $0, 4096  main  ; 7: addi $a0, $0, 4096

Controller:

Reason:

Opcode		= 001000 in base 2 = 8 in base 10 = addi in I-type

Funct			= 000000 in base 2, but not used in addi

RegDst			= 0, Rt is selected instead of Rd

MemtoReg		= 0, ALU_result is selected instead of final_RD_data

ALUCtrl			= 4’b0010, action of addition is activated in ALU

ALUSrc			= 1, se_imm ( result of sign_extension) is selected instead of Rt_data

RegWrite		= 1, final result is stored back to Rt

Jal				= 0, PC+8 is not selected

Branch			= 0

ALU

Reason:

ALUCtrl			= 4’b0010, action of addition is activated

Src1	= 0(in 32bits), value of Rs ($zero), since there is first instruction, value of all registers is initialized to 0

Src2				= 32’b0000000000000000001000000000000(4096 in base 10)

immediate after sign extension since ALUSrc = 1

ALU_result		= 32’b0000000000000000001000000000000,

Result of addition of src1 and src2

Zero			= 0, since src1 != src2

Regfile

Reason:

Read_addr_1		= 00000, address of Rs, ($zero)

Read_addr_2= 00100, address of Rt, $4= ($a0)

Read_data_1	= 0 (in 32bits) value of $zero

Read_data_2	= 0 (in 32bits) value of $4 ($a0)

RegWrite		= 1, final result is stored back to Rt

Write_addr		= 00100, address of Rt since RegDst= 0

Write_data		= 32’b0000000000000000001000000000000(4096 in base 10)

Since Jal=0, it is result of addition in ALU

Jump_Ctrl

Reason:

All Inputs are 0, therefore JumpOP =2’00

And the input of PC+4 is selected in the multiplexer(PC_Mux) to be the next address of instruction

CPU datapath

（原圖尺寸A3版本鏈接：https://drive.google.com/open?id=0B2qLIO0KTSixbG9HVW90SmVacjg）

Reason:

在Controller 里多加了用來控制 lh與sh 的 signal.架構與課本, 作業說明所給的圖大概相同

（原圖尺寸A4版本鏈接：https://drive.google.com/open?id=0B2qLIO0KTSixYkJreTBFTjl0dGM）

心得(Report)

在這次的作業中，因為上課還沒有上到這一章，所以我們需要自學來了解這次的作業。這是在這次作業中遇到最大的困難，因為我的自我學習能力很差，所以讀了很久才弄懂了整個 Single Cycle Processor 的架構跟原理。

再來的是，在寫 Verilog 的時候，也寫的很卡。而且在助教所給的範例里，與我在課本所讀到的架構長得不一樣，也因此寫錯了兩天，浪費了兩天的時間，然後透過寄信及在課堂上問助教，問題才解決。

最後的是IM_data的路徑問題，因為檔案沒有放到project 里所以無法讀取檔案。